NATIONAL

SENIOR CERTIFICATE

GRADE 12

MECHANICAL TECHNOLOGY

NOVEMBER 2009

MEMORANDUM

MARKS:  200

This memorandum consists of 16 pages.

# NB. Any other correct answer not mentioned can be accepted.

Any mathematical calculation without any unit can not be accepted as correct.

QUESTION 1:  MULTIPLE-CHOICE QUESTIONS

(Learning Outcome 3: Assessment Standards 1 – 9)

| 1.1 | C √ |  | (1) |
| --- | --- | --- | --- |

| 1.2 | A √ |  | (1) |
| --- | --- | --- | --- |

| 1.3 | D √ |  | (1) |
| --- | --- | --- | --- |

| 1.4 | C√ |  | (1) |
| --- | --- | --- | --- |

| 1.5 | B√ |  | (1) |
| --- | --- | --- | --- |

| 1.6 | D √ |  | (1) |
| --- | --- | --- | --- |

| 1.7 | A √ |  | (1) |
| --- | --- | --- | --- |

| 1.8 | C√ |  | (1) |
| --- | --- | --- | --- |

| 1.9 | C √ |  | (1) |
| --- | --- | --- | --- |

| 1.10 | B √ |  | (1) |
| --- | --- | --- | --- |

| 1.11 | C √ |  | (1) |
| --- | --- | --- | --- |

| 1.12 | D √ |  | (1) |
| --- | --- | --- | --- |

| 1.13 | D √ |  | (1) |
| --- | --- | --- | --- |

| 1.14 | C √ |  | (1) |
| --- | --- | --- | --- |

| 1.15 | A √ |  | (1) |
| --- | --- | --- | --- |

| 1.16 | C √ |  | (1) |
| --- | --- | --- | --- |

| 1.17 | D √ |  | (1) |
| --- | --- | --- | --- |

| 1.18 | C √ |  | (1) |
| --- | --- | --- | --- |

| 1.19 | B √ |  | (1) |
| --- | --- | --- | --- |

| 1.20 | A √ |  | (1) [20] |
| --- | --- | --- | --- |

#### QUESTION 2:  FORCES AND SYSTEMS AND CONTROL

(Learning Outcome 3: Assessment Standards 6 and 8)

| 2.1 | HYDRAULIC SYSTEM |  |  |
| --- | --- | --- | --- |

|  | 2.1.1 | Pressure in the system AB = πD24 = π0,124 =7,85×10−3 m2P = FBAB = 150×1037,85×10−3 = 19,11 MPa √ √ √ √ |  | (4) |
| --- | --- | --- | --- | --- |

|  | 2.1.2 | Force in piston A √ √ √ √ AA = πD24 = π0,2524 =0,49×10−3 m2P = FAAAFA = P×AA = 19,11×106×0,49×10−3 = 9,36 kN or 9360 N |  | (4) |
| --- | --- | --- | --- | --- |

|  | 2.1.3 | The distance “X” √ √ √ √ √ VA = VBVB = AB×L = 7,85×10−3×0,01 = 0,08×10−3 m3VA = AA×Χ Χ = VA AA = 0,08×10−30,49×10−3 = 0,16 m = 160 mm (Accept mm and m) |  | (5) |
| --- | --- | --- | --- | --- |

| 2.2 | Forces |  |  |
| --- | --- | --- | --- |

|  | 2.2.1 | The diameter of the brass bar √ √ √ √ √ √ √ σ = FA A = Fσ = 30×1036×106 = 5×10−3 m2A = π×D24D2 = A×4π = A×4πD = 5×10−3×4πD = 0,079788 mD = 79,788 mm = 79,79 mm |  | (6) |
| --- | --- | --- | --- | --- |

|  | 2.2.2 | The strain √ √ √ E = σεε = σEε = 6×10690×109ε = 6,666×10−5 |  | (3) |
| --- | --- | --- | --- | --- |

|  | 2.2.3 | The change in length √ √ √ ε = ΔlοlΔl = ε×οlΔl = 6,666×10−5×250Δl = 16,67×10−3 mm |  | (3) |
| --- | --- | --- | --- | --- |

| 2.3 | Belt drives (Diameter of the pump pulley) |  |  |
| --- | --- | --- | --- |

|  |  | DDN = NDR×DDRNDN = 1 440×1003 000 = 48 mm √ √ √ |  | (3) |
| --- | --- | --- | --- | --- |

| 2.4 | Wheel and axle |  |  |
| --- | --- | --- | --- |

|  | 2.4.1 | Mechanical Advantage MA = loadeffortMA = 80056 = 14,3 √ √ √ √ √ |  | (4) |
| --- | --- | --- | --- | --- |

|  | 2.4.2 | Velocity Ratio/Displacement ratio √ √√ √ VR = 2Dd2−d1VR = 2×0,30,15−0,12 = 20:1 |  | (4) |
| --- | --- | --- | --- | --- |

| 2.5 | Square threads |  |  |
| --- | --- | --- | --- |

|  | 2.5.1 | Helix angle: √ √ √ √ √√ √ Tanθ = leadπDm = 2×12π50−6 = 0,1736 θ = 9,8490 |  | (6) |
| --- | --- | --- | --- | --- |

|  | 2.5.2 | Leading angle Leading angle = 900−helix angle+clearance angle = 900−9,8490+30 = 900−12,8490 = 77,150 √ √ |  | (2) |
| --- | --- | --- | --- | --- |

|  | 2.5.3 | Trailing angle Trailing angle = 900+helix angle−clearance angle = 900+9,8490−30 = 900+6,8490 = 96,850 √ √ |  | (2) |
| --- | --- | --- | --- | --- |

| 2.6 | Clutches (Torque) |  |  |
| --- | --- | --- | --- |

√

√√

√

|  |  | T = μWnRT = 0,35×2,5×103×2 0,282 = 245 Nm |  | (4) [50] |
| --- | --- | --- | --- | --- |

#### QUESTION 3:  TOOLS AND EQUIPMENT

(Learning Outcome 3: Assessment Standard 2)

| 3.1 | Torsion |  |  |
| --- | --- | --- | --- |

|  | Torsion is the twisting action in a member caused by two opposing moments along the longitudinal axis of the member. √√ |  | (2) |
| --- | --- | --- | --- |

| 3.2 | Hook’s law |  |  |
| --- | --- | --- | --- |

|  | 3.2.1 | Strain √ is directly proportional to the stress √ its deformation causes provided the limit of proportionality/elasticity is not exceeded √ |  | (3) |
| --- | --- | --- | --- | --- |

|  | 3.2.2 | Line 0–A √ |  | (1) |
| --- | --- | --- | --- | --- |

| 3.3 | Function of tensile test |  |  |
| --- | --- | --- | --- |

|  | It is used to determine the tensile strength of material. √√ |  | (2) |
| --- | --- | --- | --- |

| 3.4 | Tensile test |  |  |
| --- | --- | --- | --- |

|  | 3.4.1 | Destructive √ |  | (1) |
| --- | --- | --- | --- | --- |

|  | 3.4.2 | Piece of material √ |  | (1) |
| --- | --- | --- | --- | --- |

|  | 3.4.3 | Axial √ |  | (1) |
| --- | --- | --- | --- | --- |

|  | 3.4.4 | Elongation √ |  | (1) |
| --- | --- | --- | --- | --- |

| 3.5 | Metal Arc Gas Shielded equipment |  |  |
| --- | --- | --- | --- |

|  | - MAGS/MIGS is a semi-automatic welding processor with a continuously fed wire from a spool √ - The wire acts as both electrode and filler when the arc is struck between the workpiece and the air. √ - A gas passing through the nozzle forms a protective shield around the welding area. √ - The arc length is self-adjusting, any variations in the arc length by the welder, produces a change in the burn-off rate of the wire √ - The arc rapidly returns to its original length. √ - The arc length is directly proportional to the voltage. √ - With a decrease in current; this causes a decrease in electrode burn-off rate √ - This restores the original arc length. √ - Steps regarding the machine setup. - No safety precautions will be accepted. |  | (8) [20] |
| --- | --- | --- | --- |

QUESTION 4:  MATERIALS

(Learning Outcome 3: Assessment Standard 3)

| 4.1 | Stainless steel |  |  |
| --- | --- | --- | --- |

|  | - With heat it does not warp - It is a tough material √ - It does not rust √ - Easy to clean √ - Neat in appearance √ - Wear resistance √ (Any 3 x 1) |  | (3) |
| --- | --- | --- | --- |

| 4.2 | Elements of stainless steel |  |  |
| --- | --- | --- | --- |

|  | - Iron √ - Carbon √ - Chrome √ |  | (3) |
| --- | --- | --- | --- |

| 4.3 | Brass |  |  |
| --- | --- | --- | --- |

|  | 4.3.1 | Elements that brass contains: - Copper √ - Zinc √ |  | (2) |
| --- | --- | --- | --- | --- |

|  | 4.3.2 | Properties: - Good resistance against corrosion √ - It is ductile and malleable √ - It can be easily cast √ - It can be easily polished √ - It can be easily chrome plated √ (Any 1x 1) |  | (1) |
| --- | --- | --- | --- | --- |

| 4.4 | Elements of solder |  |  |
| --- | --- | --- | --- |

|  | - Lead √ - Tin √ |  | (2) |
| --- | --- | --- | --- |

| 4.5 | Silver solder / Solder |  |  |
| --- | --- | --- | --- |

|  | - It has a higher melting point and a variety of applications √ - Resistance against corrosion √ - Surface preparation is easy √ |  | (2) |
| --- | --- | --- | --- |

| 4.6 | Plastics |  |  |
| --- | --- | --- | --- |

|  | - Thermoplastic √; each time they are heated they become soft and harden when cooled. Can be heated repeatedly and softened and cooled again. √ - Thermosetting; √ when heated, first time, it softens and hardens when cooled. When heated for the second time it will not soften. √ |  | (4) |
| --- | --- | --- | --- |

| 4.7 | PVC |  |  |
| --- | --- | --- | --- |

|  | Polyvinyl chloride √ |  | (1) |
| --- | --- | --- | --- |

| 4.8 | Nylon |  |  |
| --- | --- | --- | --- |

|  | - Needs no lubrications √ - Cost is low √ - Low maintenance √ - Very light in weight √ - Easy to machine √ - It’s a tough material √ - Affected by exposure to sunlight √ (Any 2 x 1) |  | (2) [20] |
| --- | --- | --- | --- |

QUESTION 5:  SAFETY, TERMINOLOGY AND JOINING METHODS

(Learning Outcome 3: Assessment Standards 1, 4 and 5)

| 5.1 | Welding |  |  |
| --- | --- | --- | --- |

|  | - Workplace should be partitioned off effectively. √ - Operator should use protective equipment. (Separate rule are acceptable) √ - Provide and maintain effective ventilation. √ - Ensure proper and adequate fire precautions. (Separate rule are acceptable) √ (Any FIVE othe correct answers are acceptable) |  | (5) |
| --- | --- | --- | --- |

| 5.2 | Centre lathe |  |  |
| --- | --- | --- | --- |

|  | - Don’t leave spanners or keys on rotary parts. √ - Use a brush or wire hook to remove shavings and not your hands. √ - Don’t adjust the gearbox of the lathe while it is running. √ - Don’t lean on the machine. √ - Don’t attempt to stop the machine by placing your hand on the chuck while the machine is slowing down. √ - Don’t make any measurements while machine is running. √ - Don’t leave machine unattended while running. √ (Any 3 x 1) |  | (3) |
| --- | --- | --- | --- |

| 5.3 | Argon or CO2 regulator Oil and grease in the presence of oxygen √ are flammable.√ |  | (2) |
| --- | --- | --- | --- |

| 5.4 | Indexing |  |  |
| --- | --- | --- | --- |

|  | 5.4.1 | Rapid indexing √ |  | (1) |
| --- | --- | --- | --- | --- |

|  | 5.4.2 | Angular indexing √ |  | (1) |
| --- | --- | --- | --- | --- |

|  | 5.4.3 | Simple indexing √ |  | (1) |
| --- | --- | --- | --- | --- |

|  | 5.4.4 | Differential indexing √ |  | (1) |
| --- | --- | --- | --- | --- |

| 5.5 | Indexing |  |  |
| --- | --- | --- | --- |

|  | Hole circles. Side 1; 24; 25; 28; 30; 34; 37; 38; 39; 41; 42; 43. Side 2; 46; 47; 49; 51; 53; 54; 57; 58; 59; 62; 66. Standard change gears. 24 x 2; 28; 32; 40; 44; 48; 56; 64; 72; 86; 100. |  |
| --- | --- | --- |

|  | 5.5.1 | Simple indexing |  |  |
| --- | --- | --- | --- | --- |

|  |  | Indexing = 40N=40119Actual indexing= 40A = 40120=4×212×2 = 824Zero full turns and 8 holes on the 24- hole circle √ √ √ √√ |  | (5) |
| --- | --- | --- | --- | --- |

|  | 5.5.2 | Change gears |  |  |
| --- | --- | --- | --- | --- |

|  |  | DrDn=A−NA×401DrDn=120−119120×401DrDn=1120×401DrDn=40120=412×66DrDn=2472 √ √ √ √√ |  | (5) |
| --- | --- | --- | --- | --- |

| 5.6 | Gears |  |  |
| --- | --- | --- | --- |

|  | 5.6.1 | Addendum = module √ = 2,5 mm √ |  | (2) |
| --- | --- | --- | --- | --- |

|  | 5.6.2 | Dedendum = 1,25 x module or =1,157 x module √ = 1,25 x 2,5 mm = 1,157x 2,5 √ = 3,125 mm = 2,8925 mm √ |  | (3) |
| --- | --- | --- | --- | --- |

|  | 5.6.3 | Cutting depth =2,25 x module or = 2,157 x module √ = 2,25 x 2,5 mm = 2,157 x 2,5 √ = 5,625 mm = 5,3925 mm √ |  | (3) |
| --- | --- | --- | --- | --- |

|  | 5.6.4 | Circular pitch =π x module √ = π×2,5 mm √ = 7,853 mm √ |  | (3) |
| --- | --- | --- | --- | --- |

|  | 5.6.5 | Clearance = 0,25 x module or = 0,157 x module √ = 0,25 x 2,5 mm = 0,157 x 2,25 √ = 0,625 mm = 0,3925 mm √ |  | (3) |
| --- | --- | --- | --- | --- |

|  | 5.6.6 | PCD=Circular pitch×number of teethπ=7,853×40π=99,987 mmORPCD=m×T=2,5×40=100 mm √ √√ √ √ √√ √ |  | (4) |
| --- | --- | --- | --- | --- |

| 5.7 | Welding defects – Weld spatter: |  |  |
| --- | --- | --- | --- |

|  | 5.7.1 | Causes: |
| --- | --- | --- |

|  |  | - Current too high. √ - Arc too long. √ - Incorrect electrode √ - Surface contamination (e.g. Rust) √ (Any 3 x 1) |  | (3) |
| --- | --- | --- | --- | --- |

|  | 5.7.2 | Prevention: |
| --- | --- | --- |

|  |  | - Correct current setting. √ - Correct welding technique. √ - Correct electrode √ - Proper surface preparation. √ (Any 3 x 1) |  | (3) |
| --- | --- | --- | --- | --- |

| 5.8 | Milling cutters |
| --- | --- |

|  | 5.8.1 | Equal-angle cutter √ |  | (1) |
| --- | --- | --- | --- | --- |

|  | 5.8.2 | Convex cutter √ |  | (1) [50] |
| --- | --- | --- | --- | --- |

QUESTION 6:  MAINTENANCE AND TURBINES

(Learning Outcome 3: Assessment Standards 7 and 9)

| 6.1 | Friction bearing failure |  |  |
| --- | --- | --- | --- |

|  | - Operating temperature too excessive. √ - Oil supply has foreign materials and /or contaminated with water. √ - Bearing material is corroded. √ - Incorrect lubricant used. √ - Lubrication insufficient. √ - Faulty design. √ - Poor maintenance and incorrect assembly. √ (Any 4 x 1) |  | (4) |
| --- | --- | --- | --- |

| 6.2 | Cutting fluid |  |  |
| --- | --- | --- | --- |

|  | - Carry away the heat generated by machining process √ - Acts as a lubricant √ - Prevents the chips from sticking and fusing to the cutter teeth √ - Improves quality of the finish of the surface √ - To keep the workpiece and the cutting tool cool √ - To obtain a high cutting speed √ - It gives a cutting tool a longer lifespan √ (Any 4 x 1) |  | (4) |
| --- | --- | --- | --- |

| 6.3 | Properties of oil |  |  |
| --- | --- | --- | --- |

|  | 6.3.1 | Viscosity of oil: |  |  |
| --- | --- | --- | --- | --- |

|  |  | Refers to the resistance √ of oil to flow. √ |  | (2) |
| --- | --- | --- | --- | --- |

|  | 6.3.2 | Pour point: |  |  |
| --- | --- | --- | --- | --- |

|  |  | Refers to the lowest temperature √ at which a liquid can flow. √ |  | (2) |
| --- | --- | --- | --- | --- |

| 6.4 | Belt slip |  |  |
| --- | --- | --- | --- |

|  | - Overloading the belt. √ - Lubricants on the contact surface. √ - Belt too slack. √ - Worn belt. √ - Contact angle on pulley too small. √ (Any 4 x 1) |  | (4) |
| --- | --- | --- | --- |

| 6.5 | Lubricating oil |  |  |
| --- | --- | --- | --- |

|  | 6.5.1 | SAE = Society of Automotive Engineers. √ |  | (1) |
| --- | --- | --- | --- | --- |

|  | 6.5.2 | 20 = Oil thin enough to be used in winter. √ |  | (1) |
| --- | --- | --- | --- | --- |

|  | 6.5.3 | W = Winter. √ |  | (1) |
| --- | --- | --- | --- | --- |

|  | 6.5.4 | 50 = Oil thick enough to be used in summer. √ |  | (1) |
| --- | --- | --- | --- | --- |

| 6.6 | V-belts |  |  |
| --- | --- | --- | --- |

|  | 6.6.1 | Advantages |  |  |
| --- | --- | --- | --- | --- |

|  |  | - V-belts are used over short distances. √ - It is silent in operation. √ - Requires very little maintenance. √ - Is able to absorb shock loads √ - Operate a low bearing pressure. √ - In a multi-V-belt drive, if one belt breaks the machine can still run on the remaining belts. √ (Any 3 x 1) |  | (3) |
| --- | --- | --- | --- | --- |

|  | 6.6.2 | Disadvantages |  |  |
| --- | --- | --- | --- | --- |

|  |  | - Cannot repair a broken V-belt √ - Unsuitable for long distances √ - V-belts have the tendency to pull tighter under heavy loads, and cause further damage when machine seizes √ - V-belts cannot be used on fixed and loose pulleys √ (Any 3 x 1) |  | (3) |
| --- | --- | --- | --- | --- |

| 6.7 | Blower |
| --- | --- |

|  | 6.7.1 | Type of blower |  |  |
| --- | --- | --- | --- | --- |

|  |  | Vane-type blower √√ |  | (2) |
| --- | --- | --- | --- | --- |

|  | 6.7.2 | Parts |  |  |
| --- | --- | --- | --- | --- |

|  |  | 1. Inlet √ 2. Outlet √ 3. Rotor √ 4. Vane √ |  | (4) |
| --- | --- | --- | --- | --- |

| 6.8 | Steam turbine – advantages |  |  |
| --- | --- | --- | --- |

|  | - It is compact. √ - No lubrication is required. √ - Steam turbine speeds can be more accurately regulated. √ - A variety of fuels can be used to obtain steam. √ - Steam turbines are more economical. √ - Higher speeds can be obtained as compared to internal combustion engine. √ - Convert heat energy into mechanical energy. √ (Any 4 x 1) |  | (4) |
| --- | --- | --- | --- |

| 6.9 | Gas turbine – advantages |
| --- | --- |

|  | - High power output from a given weight of engine. √ - The torque output characteristic permits a notable simplification of the transmission system. √ - Smooth vibration less running due to absence of reciprocating parts. √ - No rubbing parts such as piston so that internal friction and wear are almost eliminated. √ - Easy starting. √ - Can use wide range of fuels and does not require expensive anti-knock additives. √ - Low lubricating-oil consumption. √ - No water-cooling system needed. √ - Non-poisonous exhaust giving very little trouble with pollution. √ - Requires little routine maintenance √ (Any 4 x 1) |  | (4) [40] |
| --- | --- | --- | --- |

|  | TOTAL: | 200 |
| --- | --- | --- |