CVLT

# MEMORANDUM

# ISEBE LEMFUNDO LEMPUMA KOLONI

EASTERN CAPE EDUCATION DEPARTMENT

OOS-KAAP ONDERWYSDEPARTEMENT

IIMVIWO ZEBANGA LOKUGQIBELA

NATIONAL SENIOR CERTIFICATE EXAMINATIONS

NASIONALE SENIOR SERTIFIKAAT-EKSAMEN

### SEPTEMBER 2009

CIVIL TECHNOLOGY

This question paper consists of 6 pages and a 4-page answer sheet.

| QUESTION 1 |  |
| --- | --- |

| 1.1 | 1.1.1 | Haunched mortise and tenon |  | (1) |
| --- | --- | --- | --- | --- |
|  | 1.1.2 | 1.1 A – Haunch 1.1 B – Mortise |  | (2) |
|  | 1.1.3 | Where rails/stiles are provided with a groove. or When a penal is used in a frame. |  | (2) |
| 1.2 | 1.2 C |  | Comfortable height for use when standing |  |
|  | 1.2 B |  | Easy-access storage |  |
|  | 1.2 A + 1.2 E |  | Seldom used storage |  |
|  | 1.2 D |  | Frequently used storage | (5) |
| 1.3 | Enlarge gluing surface |  |  |  |
|  | Connection more sturdy |  |  |  |
|  | Widen boards |  |  |  |
|  | Lengthen boards |  |  |  |
|  | Form angles |  |  |  |
|  | Form frames (Any 4) |  |  | (4) |
| 1.4 | 1.4.1 | Lapped dovetail joint = Corner joint |  | (1) |
|  | 1.4.2 | Corner bridle joint = Framing joint |  | (1) |
|  | 1.4.3 | Corner half-lapped joint = Housed joint |  | (1) |
|  | 1.4.4 | Mortise and tenon joint = Framing joints |  | (1) |
|  | 1.4.5 | Rubbed joint = Widening joint |  | (1) |
|  | 1.4.6 | Finger joint = Lengthening joint |  | (1) |
| 1.5 | Saves time |  |  |  |
|  | Saves space |  |  |  |
|  | Prevents injuries |  |  |  |
|  | Decreases fire hazard |  |  | (4) |
| 1.6 | 1.6.1 | Position of worker - Not an unsafe position/Standing sturdy |  | (2) |
|  | 1.6.2 | Stairs - Sufficient light/Free of obstacles |  | (2) |
|  | 1.6.3 | Power supply - Switched on at control panel by means of key |  | (2) |
|  | 1.6.4 | Carrying tools with sharp ends - Sharp ends points downwards and away from body |  | (2) |
|  | 1.6.5 | Height of a stack – Not higher than 3X the width |  | (2) |
| 1.7 | Name TWO uses of each of the following tools: |  |  |  |
|  | 1.7.1 | Square nosed shovel - Earth work/Mixing of mortar |  | (2) |
|  | 1.7.2 | Flat steel square - Determining squareness/Measurements/Guide lines |  | (2) |
|  | 1.7.3 | Chalk line – Lay-out of walls/Roof lines/Plumb bob |  | (2) |
|  |  |  |  | [40] |

| QUESTION 2 |  |
| --- | --- |

| 2.1 |  |  |  |
| --- | --- | --- | --- |
| VOUSSOIRS KEY BRICK INTRADOS EXTRADOS ABUTMENT SPAN |  |  | (12) |
| 2.2 | 2.2.1 | Strength – Sufficient to support the load of wet concrete | (2) |
|  | 2.2.2 | Joints – Grout-tight for the grout not to escape | (2) |
| 2.3 | Rate of placing |  | (1) |
|  | Type of mixture |  | (1) |
|  | Method of placing |  | (1) |
|  | Air temperature |  | (1) |
| 2.4 |  | Steam Driving of enormous generators occurs by means of steam turbines. Steam can be generated by fuel as diesel, coal and nuclear power (3) |  |
|  |  | Water Driving of enormous generators occur by means of turbines driven by water. It is a cheap method to generate electricity, if the building of dams is not taken into consideration. It is called hydro electricity (3) |  |
|  |  | Wind Driving of generators occurs by means of turbines driven by wind. Wind is very intermittent in most parts of South Africa and therefore it forms a very small part of electricity supply in our country. (3) (any 3) | (9) |
| 2.5 | Red or brown – live (2) Black or blue – neutral (2) Green or yellow and green – earth (2) |  | (6) |

| 2.6 | - More consistent in strength and sturdiness because the grain of each layer runs at right angles to that of the next. (Wood is 20 to 40 times stronger along the grain than across the grain). - Higher dimensional stability. (Less expansion and shrinkage than solid wood). - A nail can be driven in close to the edge without splitting plywood. - Available in large sheets. - Economical since only the front needs to be made of expensive veneer and the rest of less expensive veneer. - Does not wrap easily (internal tension is counteracted by the cross grain). - Repetitive patterns can be obtained on the surface by using matching veneer. - Curves and bends can easily be made and are stronger (e.g. a tennis racquet). - Drying takes place more rapidly. - More durable - resistant to insects, fungi, water, weather, heat, etc. - The flexibility is much greater (especially thinner plywood). (any 5) |  | (5) |
| --- | --- | --- | --- |
|  |  |  | [40] |

| # QUESTION 3 |  |
| --- | --- |

| 3.1 | Affordability Type of building The site Ground formation Slope of site Position of site to the sun Pollution Climatic conditions Municipal regulations (any 8) |  |  | (8) |
| --- | --- | --- | --- | --- |
| 3.2 | Architect designs and draws the building. Supervises the building process. |  |  | (2) |
| 3.3 | Owner, Architect Engineer, Building surveyor Contractor, Buyer Accountant, Building manager General foreman, Trades foreman Tradesmen, Operators, Labourers (any 6) |  |  | (6) |
| 3.4 | A – Rafter B – King post C – Queen post D – Brace E – Tie – beam |  |  | (5) |
| 3.5 | Couple truss, Couple close truss, Collar truss, Couple close with post truss, King post truss, Fink truss Enlarged Fink truss, Enlarged Howe truss (any 5) |  |  | (5) |
| 3.6 | Type of roof covering |  |  | (1) |
| 3.7 | To trap greasy material to avoid blockage. Installed at a gully at the outlet of a kitchen. |  |  | (2) |
| 3.8 | Borehole, Well, Rainwater, Rivers, Fountains, Reservoirs (any 2) |  |  | (2) |
| 3.9 | 3.9.1 |  | Saves electricity. No pollution. | (2) |
|  | 3.9.2 |  | High installation cost. Must be outside. No hot water if the sun is not shining. | (2) |
| 3.10 | Manhole is installed where different sewer pipes meet. Manhole helps to clean blockages and to do inspection. |  |  | (2) |
| 3.11 | Vacuum tank/Septic tank/French drain |  |  | (3) |
|  |  |  |  | [40] |

| # QUESTION 4 |  |
| --- | --- |

| 4.1 | 1: 4x4 = 16 2: 2x8 = 16 3: 2x6 = 12 Total: = 44 |  |  | Moments around X (44xX) = (16x2)+(16x8)+(12x13) 44X = 32+128+156 = 316÷44 = 7,18 from X |  | (11) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.2 | 4.2.1 |  | SEE ANSWER SHEET |  |  | (9) |  |
|  | 4.2.2 |  | SEE ANSWER SHEET |  |  | (1) |  |
| 4.3 | 20 N 20 N 60 N 2 m 2 m 4 m A B FIGURE 4.3 |  |  |  |  |  |  |
|  | Around A LOM = ROM (Bx6)+(20x2) = (20x2)+(60x6) B6 + 40 = 40 + 360 B6 = 400 – 40 B = 360 6 = 60 N |  |  | Around B ROM = LOM (Ax6) = (20x4)+(20x8) A6 = 80 + 160 A = 240 6 = 40 N |  |  | (9) |
| 4.4 | 4.4.1 | SEE ANSWER SHEET |  |  |  | (6) |  |
|  | 4.4.2 | SEE ANSWER SHEET |  |  |  | (4) |  |
|  |  |  |  |  |  | [40] |  |

| # QUESTION 5 |  |
| --- | --- |

| 5.1 | 5.1.1 | A = r² = ( 0,005²) = 0,000 0 785 m²  = F = 400N = 5 092 958,179 Pa A 0,000 0 785 m² | (6) |
| --- | --- | --- | --- |
|  | 5.1.2 | = x = 0,000 4 = 0,000 133 Lo 3 | (3) |
|  | 5.1.3 | E =  = 5 092 958,179 Pa = 3,8210 Pa  0,000 133 | (4) |
| 5.2 | 5.2.1 | SEE ANSWER SHEET | (15) |
|  | 5.2.2 | SEE ANSWER SHEET | (2) |
| 5.3 | FOUR of the particulars below which are applicable to section elevations. |  | (4) |
|  | 5.3.1 | The width and thickness of foundations |  |
|  | 5.3.2 | The roof construction |  |
|  | 5.3.3 | Access to the site |  |
|  | 5.3.4 | Number of the site |  |
|  | 5.3.5 | Position of damp proofing |  |
|  | 5.3.6 | Ceilings and brandering |  |
|  | 5.3.7 | Building lines |  |
|  | 5.3.8 | North arrow |  |
| 5.4 | Make neat sketches to illustrate the following symbols: |  |  |
|  | 5.4.1 | Water meter | (2) |
|  | 5.4.2 | Man hole | (2) |
|  | 5.4.3 | Shower | (2) |
|  |  |  | [40] |

|  |  | TOTAL: 200 |
| --- | --- | --- |

| ANTWOORDBLAD ANSWER SHEET | A | SIVIELE TEGNOLOGIE CIVIL TECHNOLOGY | NAAM: NAME: |  |
| --- | --- | --- | --- | --- |

VRAAG/QUESTION 4.1

| Vorm/Shape | Area | X | m X |
| --- | --- | --- | --- |
| 1 | Lxb = 4x4 = 16 | b÷2 = 4÷2 = 2 | 16x2 = 32 |
| 2 | Lxb = 2x8 = 16 | b÷2 = (8÷2)+4 = 8 | 16x8 = 128 |
| 3 | Lxb = 2x6 = 12 | b÷2 = (2÷2)+12 = 13 | 12x13 = 156 |
| TOTAAL/TOTAL | = 44 |  | = 316 |
| Swaartepunt/Centre of gravity = 316÷44 = 7,18 vanaf/from X |  |  |  |

FIGURE/FIGUUR 4.2

RUIMTEDIAGRAM                                                                           KRAGTEDIAGRAM

SPACE DIAGRAM                                                                            FORCE DIAGRAM

SKAAL/SCALE: 1 mm = 1 N

60 N

A

Y

b                                              c

50 N

a

X

X = ………98 N…………..                            Y = ………92 N……………….	      d

| ANTWOORDBLAD ANSWER SHEET | B | SIVIELE TEGNOLOGIE CIVIL TECHNOLOGY | NAAM: NAME: |  |
| --- | --- | --- | --- | --- |

FIGURE/FIGUUR 4.4

| 4.4.1 | Die buigmomentwaardes/The bending moment values |
| --- | --- |

|  |  | a = …(-10x0) = 0 N……………….. |  |
| --- | --- | --- | --- |

|  |  | b = …(-10x2) = -20 N………….. |  |
| --- | --- | --- | --- |

|  |  | c = …(-10x6)+(-40x4)+(60x4) -60 – 160 + 240 = 20 N |  |
| --- | --- | --- | --- |

|  |  | d = …(-10x8)+(-40x6)+(60x6) -80 – 240 + 360 = 0 N | (6) |
| --- | --- | --- | --- |

| 4.4.2 | Die buigmomentdiagram/The bending moment diagram | (4) |
| --- | --- | --- |

SCALE/SKAAL:   2 mm = 1N

10 N            40 N                                20 N             30 N

2 m                    4 m                        2 m

60 N                                                 40 N

20 N

+

a                 b                                  c                 d

-20 N

| ANTWOORDBLAD ANSWER SHEET | C | SIVIELE TEGNOLOGIE CIVIL TECHNOLOGY | NAAM: NAME: |  |
| --- | --- | --- | --- | --- |

5 000

FIGURE/FIGUUR 5.2  (15)

4 720                      280

V1

V1

D1

Muurhoogte/Wall height: 2,6 m                                               D1 = 2 100 x 900

50 Stene/m²  vir halfsteenmuur                                                D2 = 2 100 x 1 800

50 Bricks/m² for half brick wall                                              V1 = 1 200 x 2 400

| A | B | C | D |
| --- | --- | --- | --- |
|  |  |  | Senterlyn/Centre line = 5000 + 7000 + 5000 – (2 x 280) |
|  |  |  | = 16,44 m |
| 1/ | 16,44 |  | Muurhoogte/Wall height = 2,6 m |
|  | 2,6 | 24,74 | Muuropp./Wall area = 24,74 m² |
|  | 42,74 |  | 100 stene/ m² vir spoumuur// bricks/ m² for cavity wall |
|  | 100 | 4 274 | Dus/Thus: 4 274 stene vir muur/bricks for wall |
|  |  |  | ± 40 mm |

| ANTWOORDBLAD ANSWER SHEET | D | SIVIELE TEGNOLOGIE CIVIL TECHNOLOGY | NAAM: NAME: |  |
| --- | --- | --- | --- | --- |

| A | B | C | D |
| --- | --- | --- | --- |
| 1/ | 2,1 |  | T.A. 1 x D2 = 2,1 x 1,8 |
|  | 1,8 |  |  |
|  | 100 | 378 | Dus/Thus: T.A. 378 stene vir/bricks for 1 x D2 |
|  |  |  | ± 40 mm |
| 2/ | 1,2 |  | T.A. 2 x V1 = 1,2 x 2,4 |
|  | 2,4 |  |  |
|  | 2 | 5,76 |  |
|  | 5,76 |  |  |
|  | 100 | 576 | Dus/Thus: T.A. 576 stene vir/bricks for 2 x V! |
|  |  |  | TOTALE AFTREKKINGS/TOTAL SUBTRACTIONS |
|  |  |  | 4 274 |
|  |  |  | - 378 1 x D2 |
|  |  |  | - 576 2 x V1 |
|  |  |  | 3 320 |
|  |  |  | + 166 5% vermorsing/wasting |
|  |  |  | 3 486 |
|  |  |  | DUS: Totaal van 3 486 stene benodig vir aanbouing |
|  |  |  | THUS: Total of 3 486 bricks needed for extension |