FORMULA SHEET FOR MECHANICAL TECHNOLOGY

- BELT DRIVES

1.1

1.2		Belt speed=π ( D + t )×N60 (t=belt thickness)

1.3		Belt mass= Area×length×density    (A = thickness × width)

1.4

1.5		N1D1=N2D2

1.6		Open−belt length  =  π( D+d )2+( D-d )24c+2c

1.7		Crossed−belt length = π( D+d )2+( D+d )24c+2c

1.8		Power ( P ) = 2πNT60

1.9		Ratio of tight side to slack side=T1T2

1.10		Power  =(T1−T2) π D N60where T1=force in the tight side

1.11		Width  =  T1permissible tensile force

- FRICTION CLUTCHES

| 2.1 | Torque ( T )=µWnRµ =coefficient of frictionW =total forcen =number of friction surfacesR =effective radius |
| --- | --- |

| 2.2 | Power ( P ) = 2πNT60 |
| --- | --- |

- STRESS AND STRAIN

| 3.1 | Stress = ForceArea or ( σ = FA ) |
| --- | --- |

| 3.2 | Strain ( ε ) = change in length ( ΔL )original length ( L ) |
| --- | --- |

| 3.3 | Young's modulus ( E ) = stressstrain or (σε ) |
| --- | --- |

3.4		Ashaft=πd24

3.5		Apipe=π(D2−d2)4

4. 	HYDRAULICS

4.1		Pressure ( P ) = Force ( F )Area ( A )

4.2		Volume = Cross−sectional area × stroke length ( l or s )

4.3		Work done=force  ×  distance

5.	WHEEL AND AXLE

5.1		Velocity ratio ( VR ) = effort distanceload distance = 2Dd2−d1

5.2		Mechanical advantage ( MA ) = Load ( W )Effort ( F )

5.3		Mechanical efficiency ( ηmech )=MAVR× 100%

6.	LEVERS

6.1		Mechanical advantage ( MA ) = Load ( W )Effort ( F )

6.2		Input movement ( IM ) = Effort × distance moved by effort

6.3		Output movement ( OM )=Load × distance moved by load

6.4		Velocity ratio ( VR ) = Input movementOutput movement

7.	GEAR DRIVES

7.1		Power ( P ) = 2πNT60

7.2		Gear ratio=Product of the number of teeth on driven gearsProduct of the number of teeth on driving gears

7.3		NinputNoutput=Product of the number of teeth on driven gearsProduct of the number of teeth on driving gears

7.4		Torque=force × radius

7.5		Torque transmitted = gear ratio × input torque

7.6		Module  ( m )=Pitch−circle diameter ( PCD )Number of teeth  ( T )

7.7		N1T1=N2T2

7.8		Pitch−circle diameter ( PCD )=circular pitch  ( CP )× number of teeth ( T )π

7.9		Outside diameter ( OD )=PCD+2 module

7.10		Addendum ( a ) = module ( m )

7.11		Dedendum ( b ) = 1,157 m      or     Dedendum ( b )=1,25 m

7.12		Cuttingdepth  ( h )=2,157m  or  Cutting depth ( h )=2,25 m

7.13		Clearance ( c )=0,157 m     or     Clearance ( c )= 0,25 m

7.14		Circular pitch ( CP ) = m × π

- SCREW THREADS

8.1		Pitch diameter =Outside diameter −½ pitch

8.2		Pitch circumference = π × pitch diameter

8.3		Lead = pitch × number of starts

8.4		Helix angle:  tan Ø = LeadPitch circumference

8.5		Leading tool angle = 90° – (helix angle + clearance angle)

  - Following/Trailing angle = 90° + (helix angle – clearance angle)

  - Number of turns = heightlead

- CINCINNATI DIVIDING HEAD TABLE FOR THE MILLING MACHINE

| Hole circles |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Side 1 | 24 | 25 | 28 | 30 | 34 | 37 | 38 | 39 | 41 | 42 | 43 |
| Side 2 | 46 | 47 | 49 | 51 | 53 | 54 | 57 | 58 | 59 | 62 | 66 |

| Standard change gears |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 x 2 | 28 | 32 | 40 | 44 | 48 | 56 | 64 | 72 | 86 | 100 |

9.1		Simple indexing=40n (where n=number of divisions)

9.2	Change gears: DrDv=( A−n )×40AorDrDv=(A−n)A×401

or

DrDv=(N−n)×40N

10.	CALCULATIONS OF FEED

10.1		Feed (f) = f1 T  N

Where:	f = feed in millimeters per minute

f1 = feed per tooth in millimeters

T = number of teeth on cutter

N = number of revolutions of cutter per minute

10.2	Cutting speed (V) = π×D×N

Where:  D = diameter of the cutter in meters