MCHT

MEMORANDUM

# ISEBE LEMFUNDO LEMPUMA KOLONI

EASTERN CAPE EDUCATION DEPARTMENT

OOS-KAAP ONDERWYSDEPARTEMENT

IIMVIWO ZEBANGA LOKUGQIBELA

NATIONAL SENIOR CERTIFICATE EXAMINATIONS

NASIONALE SENIOR SERTIFIKAAT-EKSAMEN

### SEPTEMBER 2009

MECHANICAL TECHNOLOGY

This memorandum consists of 17 pages.

| QUESTION 1: MULTIPLE-CHOICE QUESTIONS |  |
| --- | --- |

| (LEARNING OUTCOME 3: ASSESSMENT STANDARDS 1 – 9)√ |  |
| --- | --- |

| 1.1 | C √ |  | (1) |
| --- | --- | --- | --- |
| 1.2 | C √ |  | (1) |
| 1.3 | A √ |  | (1) |
| 1.4 | A √ |  | (1) |
| 1.5 | A √ |  | (1) |
| 1.6 | C √ |  | (1) |
| 1.7 | C √ |  | (1) |
| 1.8 | C √ |  | (1) |
| 1.9 | A √ |  | (1) |
| 1.10 | B √ |  | (1) |
| 1.11 | D √ |  | (1) |
| 1.12 | C √ |  | (1) |
| 1.13 | A √ |  | (1) |
| 1.14 | A √ |  | (1) |
| 1.15 | D √ |  | (1) |
| 1.16 | D √ |  | (1) |
| 1.17 | A √ |  | (1) |
| 1.18 | D √ |  | (1) |
| 1.19 | A √ |  | (1) |
| 1.20 | A √ |  | (1) |
|  |  |  | [20] |

| QUESTION 2: FORCES, SYSTEMS AND CONTROL |  |
| --- | --- |

| (LEARNING OUTCOME 3: ASSESSMENT STANDARDS 6 AND 8) |  |
| --- | --- |

| GEARS |  |
| --- | --- |

| 2.1 | 2.1.1 | Calculate the gear ratio: 1st Gear Ratio = No of teeth of driven gears No of teeth of driving gear = 36 x 38 22 x 18 = 1 368√ 396 1st gear ratio = 3,45 : 1√ | (3) |
| --- | --- | --- | --- |
|  | 2.1.2 | Calculate the output speed of the spindle: Product of number of teeth of the driving gear Speed of Driving gear Product of number of teeth of the driven gears = Speed of Driver gear = 18 x 22 x 3 000 rpm √ 38 x 36 = 868,42 rpm √ | (2) |
|  | 2.1.3 | Calculate the torque transmitted: Torque transmitted = Gear ratio x Torque supplied = 36 x 38 22 x 18 = 517,5 Nm√ | (1) |

| 2.2 | CALCULATE THE BELT DRIVES: |  |  |
| --- | --- | --- | --- |
|  | 2.2.1 | Open belt length = 2x (D + d) + (D−d)4c2 + 2c = 2x (220 + 110) + (220−110)24c + (2 x 550) mm = 2x (330) + (110)22200 + 1100 = 2x (330) + (12100)2200+ 1 100 √ = 518,43 + 5,5 + 1 100√ = 1 623,93 mm√ | (3) |
|  | 2.2.2 | Cross belt drive = 2x (D + d) + (D+d)4c2 + 2c = 2x (220 + 110) + (220+110)24c + (2 x 550) mm = 2x (330) + (330)22200 + 1 100 = 2x (330) + (108900)2200+ 1 100 √ = 518,43 + 49,5 + 1 100√ = 1 667,93 mm√ | (3) |
|  | 2.2.3 | Drive with less slipping will be the cross belt drive. | (1) |

| TORQUE: |  |  |  |
| --- | --- | --- | --- |
| 2.3 | 2.3.1 | Calculate Torque (T) = WnR = 0,35 x 3,2 x 0,08 kNm√ (1) = 0,1792 kNm√ (1) | (2) |
|  | 2.3.2 | POWER (P) Calculate Power = 2NT 60 = 2 x 2 800 x 0,1792 kW = 3 153,92 kW√ 60 Power = 52,565 Kw√ | (2) |
| THREE-START SCREW THREAD |  |  |  |
| 2.4 | 2.4.1 | Calculate the Helix angle of the thread: Helix angle tan Ø = leadpitchcircumference = 36188,49√ tan Ø = 0,191√ Ø = 10,81º√ | (3) |
|  | 2.4.2 | Calculate the leading tool angle: Leading tool angle = 90º – (Helix angle + clearance angle) = 90º – (10,81º + 3º) = 90º – 13,81√ = 76,19º√ | (2) |

|  | 2.4.3 | Calculate the trailing tool angle: Trailing tool angle = 90º + (10,81º – 3º) = 90º + 7,81º√ = 97,81º√ | (2) |
| --- | --- | --- | --- |
| 2.5 | STRESS AND STRAIN (YOUNG'S MODULUS) |  |  |
|  | 2.5.1 | Calculate the total length mm or m Total Length = 350 + 150 √ = 500 mm or 0,5 m √ | (2) |
|  | 2.5.2 | Calculate the stress at Section A and B: AA = π D2A 4 = π 0,052 4 = 1,96 10−3 m2 √ (1) σA = F AA = 150 x 103 1,96 x 10−3 = 76,53 x 106 = 76,53 MPa √ (2) AB = π D2B 4 = π 0,042 4 = 1,26 x 10−3 m2 √ (1) σB = F AB = 150 x 103 1,26 x 10−3 m2 = 119,05 x 106 Pa √ 119,05 MPa √ (2) | (6) |

|  | 2.5.3 | Calculate the strain induced in Section A and B: ε A = σAE =76,53x10680x109√ = 0,96 10-3 εB = σBE = 119,05x10680x109√ = 1,49 x 10−3 √ (2) |  |
| --- | --- | --- | --- |
|  | 2.5.4 | Calculate the total final length of the pin: Final Length Total length = L L mm εA = Δ LA OLA Δ LA = εA x OLA = 0,96 x10−3 x 350√ = 0,34 mm √ (2) εB = Δ LB OLB Δ LB = εB x OLB = 1,49 x10−3 x 150 √ = 0,22 mm √ (2) FINAL LENGTH = TOTAL LENGTH + Δ LA + Δ LB = 500 + 0,34 + 0,22 = 500, 56 mm √ (1) | (5) |

| 2.6 | HYDRAULICS |  |  |
| --- | --- | --- | --- |
|  | 2.6.1 | Calculate the pressure applied to piston A: Pressure at piston A = Pressure at piston B Area of piston A = πd2 4 = xo,1524 = 0,00177 m2 √ (1) Area at piston B = πd2 4 = x0,7524 = 0,44 m2 √ (1) Force applied at piston B = FA = 4500,442Pa √ = 1 018,1 Pa √ (2) Pressure at piston B equal pressure at piston A Pressure at A = force at A ∴PA = FA AA ∴FA = PA X AA = 1018,1 x 0,0177 N = 18,02 N √ (2) | (6) |

|  | 2.6.2 | Volume displaced by piston B = AB x LB = 0,442 x 0,012 m3 = 0,0053 m3√ | (2) |
| --- | --- | --- | --- |
|  | 2.6.3 | Length or distance moved by piston A Volume A = AA x LA LA = 0,00530,0177√ = 0,299 m = 299 mm√ | (2) |
|  |  |  | [50] |

| QUESTION 3: TOOLS AND EQUIPMENT |  |
| --- | --- |

| 3.1 | SPRING TESTER |  |  |
| --- | --- | --- | --- |
|  | 3.1.1 | Scale |  |
|  | 3.1.2 | Indicator |  |
|  | 3.1.3 | Upper platform |  |
|  | 3.1.4 | Lower platform |  |
|  | 3.1.5 | Compressing lever | (5) |
| 3.2 | 3.2.1 | Tailstock√ |  |
|  | 3.2.2 | To support work turning between the two centre One centre is placed in the headstock spindle While the other centre accommodated in the nose of the tailstock spindle. √ |  |
|  | 3.2.3 | A drilling operation can be performed by mounting the drill in the nose of the tailstock spindle.√ |  |
|  | 3.2.4 | A taper turning operation can be carried out on a work piece mounted between centres. The tailstock centre is moved out of line with the centre of the lathe spindle.√ | (4) |
| 3.3 | Continuity test: |  |  |
|  | 3.3.1 | Connect the red test lead to the VWm-A jack and the black test lead to the COM jack√ |  |
|  | 3.3.2 | Set the range switch to the diode range.√ |  |
|  | 3.3.3 | Connect the red test lead to the anode of the diode to the tested, and the black test lead to the cathode of the diode. √ |  |

|  | 3.3.4 | The approximate forward voltage drop of the diode will be displayed in M.v. If the connection is reversed only figure ‘1’ will be shown on the LCD. √ |  |
| --- | --- | --- | --- |
|  | 3.3.5 | Connect the test leads to the two terminals of the circuit to be tested if the resistance is less than about 50Ω, the buzzer will sound. √ | (5) |
| 3.3 | PRESSURE TESTER is used to determine the test pressure of a pressure vessel. √ If there is a pressure drop after 20 – 30 minutes it means that there is a leakage in the valve or seams. √ The tests are carried on vessels containing either air or fluids. √ CYLINDER LEAKAGE TESTER is used to determine any leak through air intake valve, an exhaust valve, head or block and excessive leakage past the piston rings. √ If the tester shows a pressure drop after 20 – 30 min it means that there is a leakage in the cylinder. √ The tests are carried out only on internal combustion engines.√ |  | (6) |
|  |  |  | [20] |

| QUESTION 4: MATERIALS |  |
| --- | --- |

| (LEARNING OUTCOME 3: ASSESSMENT STANDARD 3) |  |
| --- | --- |

| 4.1 | MATERIALS |  |  |
| --- | --- | --- | --- |
|  | 4.1.1 | A non-ferrous alloy is a metal that has a combination of two or more non-ferrous metals, which are melted together to form one alloy. | (2) |
|  | 4.1.2 | Composites are materials that are made up of two or more different materials put together. | (2) |

| 4.2 | ENGINEERING MATERIALS |  |
| --- | --- | --- |

| MATERIAL | PROPERTIES | USES |
| --- | --- | --- |
| 4.2.1 White metal | Any TWO of the following: - Low friction - Low melting - Casts well | Any TWO of the following: - Big-end bearings - Main bearings - Bearings for electrical machines |

|  |  |  | (4) |
| --- | --- | --- | --- |

| 4.2.2 Vanadium | Any TWO of the following: - Wear resistant - Fatigue resistant - Makes steel stronger | Any TWO of the following: - Crankshafts - Connecting rods - Gears - Vehicle - Steering components |
| --- | --- | --- |

|  |  |  | (4) |
| --- | --- | --- | --- |

| 4.2.3 Nylon | Any TWO of the following: - Needs no lubrication - Can withstand a lot of shock - No maintenance - Light in weight - Easy to machine | Any TWO of the following: - Fan blades - Bearings - Gears - Trolley wheels - Bolts and nuts |
| --- | --- | --- |

|  |  |  | (4) |
| --- | --- | --- | --- |

| 4.3 | BRASS - It is resistant against corrosion - It is ductile and malleable - Can be polished to give a good shine - It is softer than mild steel and easy to machine | (4) |
| --- | --- | --- |
|  |  | [20] |

| QUESTION 5: SAFETY, TERMINOLOGY AND JOINING METHODS |  |
| --- | --- |

| 5.1 | LATHE MACHINE – SAFETY |  |  |
| --- | --- | --- | --- |
|  | - Clamp work piece solidly. - Use the correct size tool and work-holding device for the job. - Get an assistant when working with heavy chunks, attachments and work piece. - Make sure that all guards are in place before you operate the machine. - Stop the machine before making measurements or adjustments. - Do not attempt to remove chips with your hand. Use a brush or short stick. - Remove neck tie, wrist watch and jewellery. - Always remove the key from the chuck. - Wear apron and safety goggles when working on a lathe. - Tools must not be placed on the lathe ways. (Any 5 x 1) |  | (5) |
| 5.2 | METAL ARC GAS SHIELDED SAFETY OR METAL INERT GAS WELDING SAFETY. |  |  |
|  | - The work area must be well ventilated because of toxic gases. - The work area must be well illuminated. - Wear flameproof gauntlet, gloves, flameproof apron or leather, asbestos or other non-flammable materials, because of hot sparks. - Do not use gloves to carry hot metal, use a pair of pliers. - Use fire-resistant leggings and safety boots to protect legs and feet. - When performing overhead welding, the operator must use the face sheet and helmet. (Any 4 x 1) |  | (4) |
| 5.3 | MILLING CUTTERS |  |  |
|  | 5.3.1 | Plain helical cutter √ |  |
|  | 5.3.2 | Side and face cutter √ |  |
|  | 5.3.3 | Slotting cutter √ |  |
|  | 5.3.4 | Dove tail cutter √ |  |
|  | 5.3.5 | Slot drill √ | (5) |

| 5.4 | Indexing |  |
| --- | --- | --- |

| INDEX PLATE HOLE CIRCLES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Side1 | 24 | 25 | 28 | 30 | 34 | 37 | 38 | 39 | 41 | 42 | 43 |
| Side2 | 46 | 47 | 49 | 51 | 53 | 54 | 57 | 58 | 59 | 62 | 66 |

| STANDARD CHANGE GEARS |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 × 2 | 28 | 32 | 40 | 44 | 48 | 56 | 64 | 72 | 86 | 100 |

|  | 5.4.1 | INDEXING: Use A = 86 and n = 87 Indexing = 40 ÷ A = 40 ÷ 87→no calculation possible choose A = 86 Indexing = 40 ÷ A √ = 40 ÷ 86 √ = 20 ÷ 43 √ i.e. no full turns and 20 holes in a 43 hole circle plate. | (3) |
| --- | --- | --- | --- |
|  | 5.4.2 | Change gears: Dr ÷ Dv (gear ratio) = (A/N−n) x 40 ÷ A/N √√ = (86 − 87) x 40 ÷ 86√√ = -1 x 40 ÷ 86√ Dr ÷ Dv = -40 ÷ 86√ | (6) |
|  | 5.4.3 | Meaning of positive (+) and negative (−) signs If the sign is positive (+) when determining change gears, the rotation of the index plate will be in the same direction of rotation of the crank handle. √√ If the sign is negative (−) when determining change gears, the rotation of the index plate will be in the opposite direction of rotation of the crank handle.√√ | (4) |
| 5.5 | Calculate the feed on a milling machine: D = 80 ÷ 1000 = 0,08 m√ V = πDN N = V ÷ πD = 25 ÷ π x 0,08√ = 99,47 r/min√ F= f1 x T x N = 0,04 x 16 x 99,47√ = 63,66 mm/min√ = 64 mm/min √ |  | (6) |
| 5.6 | Liquid dye penetrate - Clean the surface to be tested. √ - A liquid dye penetrate is sprayed onto the clean surface. √ - Allow a short time for the dye to penetrate the weld joint. √ - Use a cloth to remove excess dye on the weld. √ - Wash the surface and allow to dry thoroughly. √ - Use a developer and spray on the surface which brings out the colour in the dye penetrate that has penetrated into the cracks or pinholes. √ |  | (6) |
| 5.7 | Destructive tests on a welded joint - To check whether proper welding procedures were followed during the welding process.√ - To check for visual weld defects.√ |  | (2) |

| 5.8 | WELDING DEFECTS |  |
| --- | --- | --- |

| WELD DEFECT | CAUSES | CORROSION METHOD |
| --- | --- | --- |
| Poor appearance | Any ONE of the following: - Current too high or too low - Incorrect use of electrode - Faulty electrode | Any ONE of the following: - Adjust current to required amperage - Check the procedure of welding - Change the electrode or use dry electrode |
| Weld crater | Any ONE of the following: - Current too high - Improper welding technique - Electrode too small | Any ONE of the following: - Use lower current - Use proper welding technique - Use correct electrode |
| Slag inclusion | Any ONE of the following: - Included angle too narrow - Rapid chilling - Weld temperature too low - High viscosity of molten metal | Any ONE of the following: - Preheat metal - Remove slag from previous run weld - Increase the included angle |
| Porous weld | Any ONE of the following: - Speed too fast - Current too low - Insufficient pudding time - Faulty electrode - High sulphur or other impurities in metal - Impaired base metal - Short arc with exception of low hydrogen and stainless | Any ONE of the following: - Use correct current - Hold a longer arc - Use low hydrogen electrodes - Check for impurities in base metal - Allow for sufficient pudding time for gases to escape - Weave the weld - Use correct electrode for the job |

| Incomplete penetration | Any ONE of the following: - Speed too fast - Joint design faulty - Electrodes too large - Current too low | Any ONE of the following: - Use correct current to obtain desired penetration - Weld slowly - Calculate the electrode penetration properly - Select correct electrode according to welding groove - Leave enough free space at the bottom of the weld |
| --- | --- | --- |
| Undercutting | Any ONE of the following: - Faulty electrode manipulation - Current too high - Arc length too low - Speed of weld too fast | Any ONE of the following: - Use a uniform weave in butt welding - Do not use a too large electrode - Avoid excessive weaving - Current to be moderated and weld slowly - Hold the electrode at a safe distance from the vertical plane when making a horizontal fillet weld |

|  |  | (Any 3 x 3) | (9) |
| --- | --- | --- | --- |
|  |  |  | [50] |

| QUESTION 6: MAINTENANCE AND TURBINES |  |
| --- | --- |

| 6.1 | 6.1.1 | Viscosity must be correct |  |
| --- | --- | --- | --- |
|  | 6.1.2 | It must resist corrosion |  |
|  | 6.1.3 | It must prevent rust |  |
|  | 6.1.4 | It must avoid forming scum |  |
|  | 6.1.5 | Resist carbon scum forming |  |
|  | 6.1.6 | Must prevent oxidation |  |
|  | 6.1.7 | Must resist extreme pressures and temperature (Any 4) | (4) |

| 6.2 | Bearing failures - Insufficient lubrication - Excessive lubrication - Grease liquefaction - Foaming oil - Abrasive or corrosive contaminants in bearing - Raceway turning on shaft or in housing - In adequate bearing clearances caused by using to tight on Shaft or on housing - Excessive clearance - Contamination /dirt (Any 4 x 1) | (4) |
| --- | --- | --- |
| 6.3 | Viscosity, Viscosity Index, load-carrying capacity, FZG test Pour point. Rust and corrosion protection, Demulsifibility, Water separation, Anti-foaming, Oxidation resistance. (Any 4) | (4) |

| 6.4 | Draining and filling gearbox oil procedure. - It is advisable to drain the gearbox oil immediately after the car has been driven, that is when the oil is warm. Oil will flow freely. - Locate the filler plug on the side of the gearbox casing and wipe the plug and the area around it clean. Place the tray under the gearbox. - Ensure that you use the correct spanner and loosen the plug. - Remove the filler plug. - Remove the drain plug by fitting the spanner over the plug and turning it loose. - Allow oil to drain out of the gearbox into the drain pan. - Clean the drain plug, fit new seal to plug. - Replace the drain plug and tighten. - Refill the gearbox with new oil. - Allow excess oil to trickle out and refit the filler plug | (10) |
| --- | --- | --- |
| 6.5 | Roots blower √ (1) 6.5.1 Air inlet 6.5.2 Roots blower 6.5.3 Inlet port 6.5.4 Exhaust valve 6.5.5 Piston 6.6.6 Injector (6) | (7) |

| 6.6 | ENGINE BLOWER |  |  |
| --- | --- | --- | --- |
|  | 6.6.1 | Centrifugal blower √ | (1) |
|  | 6.6.2 | 1 = Air inlet 2 = Air outlet 3 = Impeller 4 = Fins and blades (4 x 1) | (4) |
|  | 6.6.3 | Advantages: - Increases the output power - A smaller engine fitted with a centrifugal blower delivers the same power as a larger engine - It eliminates lack of oxygen above sea level - Increases the volumetric efficiency of the engine - With the aid of the intercooler both the power and the torque output of the engine are increases (Any 2) | (2) |
| 6.7 | Corrosion and rust resistance of oil: - It is the ability of oil displace water from the metal allowing the oil to coat the surface √ - It also has the alkaline reaction to neutralise combustion acid thus preventing corrosion. √ - Viscosity is the resistance of oil to flow under different temperatures, √ - It should also maintain a hydro-dynamic lubricating film and be resistance to rupture under heavy loads √ |  | (4) |
|  |  |  | [40] |
|  |  | TOTAL: 200 |  |